Instituto Politécnico Nacional

Escuela Superior de Computo

Materia: Bases de Datos

Profesor: Iván Diaz Toala

Equipo:

Oswaldo Gil Valentín

Rojas Tapia Italuu

Godínez Pérez Karla

Grupo: 3CV4                     Fecha: 10/12/2023

Actividad: Reporte Practica 4

Descripción:

Un convertidor de binario a 7 segmentos es un circuito lógico utilizado para convertir un número binario (normalmente de 4 bits, representando números del 0 al 15) en una señal adecuada para mostrarlo en un display de 7 segmentos.

Cálculos de diseño:

| N | A | B | C | D | a | b | c | d | e | f | g |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 3 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 4 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 5 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 6 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 7 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 8 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 9 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 |
| 10 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 11 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| 12 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| 13 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| 14 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| 15 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |

a(A,B,C,D)∑(0,2,3,5-10,12,14,15)

b(A,B,C,D)∑(0-4,7-10,13)

c(A,B,C,D)∑(0,1,3-11,13)

d(A,B,C,D)∑(0,2,3,5,6,8,11-14)

e(A,B,C,D)∑(0,2,6,8,10-15)

f(A,B,C,D)∑(0,4-6,8-12,14,15)

g(A,B,C,D)∑(2-6,8-11,13-15)

a(A,B,C,D)∑(0,2,3,5-10,12,14,15)       b(A,B,C,D)∑(0-4,7-10,13)

c(A,B,C,D)∑(0,1,3-11,13)                   d(A,B,C,D)∑(0,2,3,5,6,8,11-14)

e(A,B,C,D)∑(0,2,6,8,10-15)                         f(A,B,C,D)∑(0,4-6,8-12,14,15)

g(A,B,C,D)∑(2-6,8-11,13-15)

Código VHDL:

library ieee;

use ieee.std_logic_1164.all;

entity BIN7SEG is

port(A, B, C, D: in std_logic;

F1: out std_logic;

F2: out std_logic;

F3: out std_logic;

F4: out std_logic;

F5: out std_logic;

F6: out std_logic;

F7: out std_logic);

end BIN7SEG;

architecture BIN7SEG of BIN7SEG is begin

F1 <=((NOT B) AND (NOT D)) OR (B AND C) OR ((NOT A) AND C) OR ((NOT A) AND B AND D) OR (A AND (NOT B) AND (NOT C)) OR (A AND (NOT C) AND (NOT D));

F2 <= ((NOT B) AND (NOT D)) OR ((NOT B) AND (NOT C)) OR (A AND (NOT C) AND D) OR ((NOT A) AND (NOT C) AND (NOT D)) OR ((NOT A) AND C AND D) OR ((NOT A) AND (NOT B));

F3 <=(A AND (NOT B)) OR ((NOT A) AND B) OR ((NOT C) AND D) OR ((NOT A) AND (NOT C)) OR ((NOT A) AND D);

F4 <=((NOT B) AND (NOT C) AND (NOT D)) OR (A AND B AND (NOT C)) OR (B AND (NOT C) AND D) OR ((NOT A) AND (NOT B) AND C) OR ((NOT B) AND C AND D) OR (B AND C AND (NOT D));

F5 <=((NOT B) AND (NOT D)) OR (A AND B) OR (C AND (NOT D)) OR (A AND C);

F6 <=(A AND (NOT B)) OR (A AND C) OR ((NOT C) AND (NOT D)) OR (B AND (NOT D)) OR (A AND B AND (NOT C));

F7 <=((NOT A) AND B AND (NOT C)) OR (A AND (NOT B)) OR (C AND (NOT D)) OR (A AND D) OR ((NOT B) AND C);

end BIN7SEG;

Evidencias de prueba del circuito (fotografías de circuito y su funcionamiento):