Instituto Politécnico Nacional

Escuela Superior de Computo

Materia: Fundamentos de Diseño Digital

Profesor: Iván Diaz Toalá

Equipo:

Oswaldo Gil Valentín

Rojas Tapia Italuu

Godínez Pérez Karla

Grupo: 3CV4                     Fecha: 10/12/2023

Actividad: Reporte Practica 5

Descripción:

Un convertidor BCD a 7 segmentos es un circuito lógico que toma un número en formato BCD (Binary-Coded Decimal) y lo convierte en una señal adecuada para mostrarlo en un display de 7 segmentos. El formato BCD representa cada dígito decimal con su equivalente binario de 4 bits.

Cálculos de diseño:

|  | Entradas |  |  |  | Salidas |  |
| --- | --- | --- | --- | --- | --- | --- |
| N | A | B | C | D | Caso 1 | Caso 2 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 2 | 0 | 0 | 1 | 0 | 1 | 1 |
| 3 | 0 | 0 | 1 | 1 | 1 | 1 |
| 4 | 0 | 1 | 0 | 0 | 0 | 0 |
| 5 | 0 | 1 | 0 | 1 | 1 | 1 |
| 6 | 0 | 1 | 1 | 0 | 0 | 0 |
| 7 | 0 | 1 | 1 | 1 | 1 | 1 |
| 8 | 1 | 0 | 0 | 0 | 0 | 0 |
| 9 | 1 | 0 | 0 | 1 | 0 | 0 |
| 10 | 1 | 0 | 1 | 0 | X | X |
| 11 | 1 | 0 | 1 | 1 | X | X |
| 12 | 1 | 1 | 0 | 0 | X | X |
| 13 | 1 | 1 | 0 | 1 | X | X |
| 14 | 1 | 1 | 1 | 0 | X | X |
| 15 | 1 | 1 | 1 | 1 | X | X |

a(A,B,C,D)=∑(0,2,3,5-9) + β(10-15)

b(A,B,C,D)=∑(0-4,7-9) + β(10-15)

c(A,B,C,D)=∑(0,1,3-9) + β(10-15)

d(A,B,C,D)=∑(0,2,3,5,6,8) + β(10-15)

e(A,B,C,D)=∑(0,2,6,8) + β(10-15)

f(A,B,C,D)=∑(0,4-6,8,9) + β(10-15)

g(A,B,C,D)=∑(2-6,8,9) + β(10-15)

a(A,B,C,D)=∑(0,2,3,5-9) + β(10-15)    b(A,B,C,D)=∑(0-4,7-9) + β(10-15)

c(A,B,C,D)=∑(0,1,3-9) + β(10-15)    d(A,B,C,D)=∑(0,2,3,5,6,8) + β(10-15)

e(A,B,C,D)=∑(0,2,6,8) + β(10-15)     f(A,B,C,D)=∑(0,4-6,8,9) + β(10-15)

g(A,B,C,D)=∑(2-6,8,9) + β(10-15)

Código VHDL:

library ieee;

use ieee.std_logic_1164.all;

entity BCD7SEG is port(A, B, C, D: in std_logic;

F1: out std_logic;

F2: out std_logic;

F3: out std_logic;

F4: out std_logic;

F5: out std_logic;

F6: out std_logic;

F7: out std_logic);

end BCD7SEG;

architecture BCD7SEG of BCD7SEG is begin

F1 <=((NOT B) AND (NOT D)) OR (B AND D) OR (C) OR (A);

F2 <=((NOT C) AND (NOT D)) OR (C AND D) OR (NOT B);

F3 <=(NOT C) OR (D) OR (B);

F4 <=((NOT B) AND (NOT D)) OR ((NOT B) AND C) OR (C AND (NOT D)) OR (B AND (NOT C) AND D);

F5 <=((NOT B) AND (NOT D)) OR (C AND (NOT D));

F6 <=(A) OR (B AND (NOT C)) OR (B AND (NOT D)) OR ((NOT C) AND (NOT D));

F7 <=(A) OR (B AND (NOT C)) OR ((NOT B) AND C) OR (C AND (NOT D));

end BCD7SEG;

Evidencias de prueba del circuito (fotografías de circuito y su funcionamiento):

Simulaciones en VHDL Sim: